COMPRESSOR EQUIVALENCE

IDENTIFY YOUR COMPRESSORS

| CODE |  |  |  |  |  |  |  |  | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | AJ | 4 | 5 | 19 | Z | H | R |  | Group description |
| T | AJ | 4 | 5 | 19 | Z |  |  |  | Compressor description |
|  |  |  |  |  |  |  | R |  | No letter = welding valve Letter R = with bottle |
|  |  |  |  |  |  | H |  |  | H = high induction pressure (-15°C à + 15°C) B = low induction pressure (-40°C à -10°C) M = medium and high induction pressure (-25°C à +15°C) |
|  |  |  |  |  | Z |  |  |  | A, B, or D = refrigerant R12 C = refrigerant R407 E, F, G or H = refrigerant R22 J, K, L = refrigerant R502 T = refrigerant R22 or R502 Y = refrigerant R134A Z = refrigerant R404A or R407B or R507 |
|  |  |  |  | 19 |  |  |  |  | Corresponds to the first refrigeration power numbers expressed in BTU/h at 60Hz Example : 19 preceded by the number 5 means 19 000 BTU/h |
|  |  |  | 5 |  |  |  |  |  | Number of figures composing the refrigeration power Example : 19 000 BTU/h |
|  |  | 4 |  |  |  |  |  |  | Applications 1 = Low vaporization pressure. Engine with standard start-up torque 2 = Low vaporization pressure. Engine with start-up torque 3 = High vaporization pressure. Engine with standard start-up torque 4 = High vaporization pressure. Engine with start-up torque 5 = Air conditioning 9 = Medium and high vaporization pressure. Engine with high start-up torque 0 = Medium and high vaporization pressure. Engine with high standard start-up torque |
|  | AJ |  |  |  |  |  |  |  | Family : TH-AZ-AEZ-AE-AJ-FH-AH-AN-RK-RGA P designates compressors intended for parallel disassembly Ex : TAHP, TAGP, TANP D designates compressors mounted in 'couple' S designates single-propeller condensation groups |
| T |  |  |  |  |  |  |  |  | C or noletter = single-phased T = tri-phased |

R12-POWERED LOW PRESSURE COMPRESSORS

| Hermetic Unit | CV | Aspera | Danfoss | Necchi | Uninad | Zanussi | DKK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AE12Z7 | 1/12 | A1055 | TL3A | M3,5 | D30AS | E30601 | KH0,63N06 |
| AZ1335D | 1/8 | A1075 | TL4A | M4 | D36AS | E44101 | KH0,75N07 |
| AZ1355D | 1/6 | A1111 | TL5A | M5 | L55A | V1040G | MH1,0N12 |
| AEZ1360A | 1/5 | B1112 | FR7,5A | M7 | L76A | V1350E | MH1,25N12 |
| AEZ1380A | 1/4 | A1118 | FR10A | M9 | L88 | E88102 | MH1,6H15 |
| AE1410A | 1/3 | E1121 | FR11A | M11 | P12 |  | MH2,0H15 |
| AE1412A | 3/8 | T1124 | SC15A | M13 | P14 |  |  |

This table is published for your guidance only, on no account shall SOGAFRIC SERVICES be held liable for it.

R12-POWERED HIGH PRESSURE COMPRESSORS

| Hermetic Unit | CV | Aspera | Danfoss | Necchi | Uninad | DKK |
| --- | --- | --- | --- | --- | --- | --- |
| AZ9387 | 1/6 | A5132 | FR6BX | M4H | AE65ZA7 |  |
| CAE59ZF9 | 1/5 | A5160 | FR8,5B | M7H | L57PX | MH1,25N18 |
| CAE4440A | 1/3 | E6185 | SC10B |  | P14TY |  |
| CAJ4452A | 3/8 | E6187 | SC12B |  |  |  |
| CAJ4461A | 1/2 | T6213 | SC15B |  | CAJ2P |  |
| CAJ4492A | 3/4 | J6220 | SC21B |  |  |  |
| CAJ4511A | 1 | J6226 |  |  |  |  |
| CAH4518A | 1,5 | H6238 |  |  |  |  |
| CAH4525A | 2 | H6253 |  |  |  |  |

This table is published for your guidance only, on no account shall SOGAFRIC SERVICES be held liable for it.